AIRBORNE ELECTRONICS 350A

Digital Replacement Governor

Bill of Materials, System Architecture & Software Specification

Beechcraft Bonanza Electric Prop — Beech 215-210 Motor / Airborne 350A STC SA2700WE

⚠ REGULATORY NOTE: This document is for engineering and planning purposes. Any modification to certified aircraft avionics constitutes a major alteration and requires FAA Form 337 approval by an IA holding appropriate ratings. The system described here uses the identical external interface (7-pin connector, existing relays, airframe wiring) as the original 350A, which supports the 337 documentation case. Review with your IA before proceeding.

# 1. Design Philosophy & Improvements Over Original 350A

The original Airborne Electronics 350A is a bang-bang (on/off) controller. It has no proportional response — the prop motor runs at full voltage until the RPM crosses the setpoint, then reverses. This architecture causes the hunting behavior that worsens with a freshly overhauled, higher-torque motor because the motor overshoots before the controller can react.

The digital replacement preserves the complete external interface (the 7-pin Cannon connector, the two airframe relays, the prop motor, and all limit switches) and replaces only the electronics inside the governor box. From the airframe's perspective, nothing changes. The improvements are entirely internal to the control algorithm.

## Key Improvements

- Proportional control — motor runs at reduced PWM duty cycle as actual RPM approaches setpoint, eliminating overshoot

- Configurable deadband — software-defined quiet zone (default ±25 RPM) where no correction is applied; sized to match your specific motor torque

- Averaged RPM measurement — magneto pulse interval measured directly by hardware timer, averaged over multiple pulses; eliminates capacitor-aging noise

- Clean setpoint input — ADC readings averaged over 10ms; eliminates dirty-pot noise spikes from triggering corrections

- Linear software mapping — regardless of pot taper, software maps the 0–10kΩ range to RPM linearly

- Named presets with numeric display — five configurable RPM setpoints selectable by pushbutton; active setpoint shown on OLED display

- Watchdog timer — if firmware locks up, relays de-energize within 250ms; prop coasts to governor-neutral

- Motor run-time limiter — relay cannot be held energized more than 8 seconds continuously without an RPM change confirmation; prevents runaway if limit switch fails

- Loss of magneto signal detection — if no pulses received for 2 seconds, system halts relay outputs and flags alarm on display

- Power-on safe state — system defaults to MANUAL (relays de-energized) at power-on; AUTO must be explicitly selected

# 2. System Architecture

## Two-Unit Architecture

The system splits into two units connected by a multi-conductor cable:

- Governor Box (replaces 350A housing): MCU, power supply, relay drivers, RPM input optocoupler. Mounts in the same location as the original, uses the same 7-pin Cannon connector.

- Panel Unit (replaces the rheostat knob): OLED display, 5 preset pushbuttons, rotary encoder for fine trim. Mounts in the existing prop control panel opening.

## Signal Flow

RIGHT MAGNETO P-LEAD ¦ [10kΩ isolating resistor + 6N137 optocoupler] ← galvanic isolation, protects MCU ¦ MCU TIMER INPUT CAPTURE PIN ← measures pulse interval in microseconds ¦ RPM CALCULATION (RPM = 60,000,000 / (pulse_interval_µs × pulses_per_rev)) ¦ CONTROL LOOP ← compare actual RPM to setpoint, apply deadband, compute correction ¦ ┌────────────┐ ┌────────────────┐ │ INC RPM │ ──────▶ │ ULN2003A │ ──▶ Lead C ──▶ INC RPM RELAY ──▶ Motor │ DEC RPM │ ──────▶ │ Darlington │ ──▶ Lead D ──▶ DEC RPM RELAY ──▶ Motor └────────────┘ └────────────────┘ ¦ PANEL UNIT ──▶ I2C ──▶ OLED DISPLAY (presets, encoder)

## Connector Pin Assignment (7-pin Cannon — unchanged from original)

| Pin | Original Use | New Use | Notes |
| --- | --- | --- | --- |
| A | 12V supply | 12V supply input | Through prop circuit breaker and AUTO switch |
| B | Airframe ground | Airframe ground | Governor box ground — bond to box enclosure |
| C | INC RPM relay drive (output) | INC RPM relay drive (output) | 12V when RPM too low; drives existing airframe relay coil |
| D | DEC RPM relay drive (output) | DEC RPM relay drive (output) | 12V when RPM too high; drives existing airframe relay coil |
| E | Rheostat wiper end 1 | Setpoint input (ADC) | 0–5V from panel pot; read by MCU ADC |
| F | Rheostat wiper end 2 | Setpoint input (ADC) | Other end of pot; one end to GND, other to +5V |
| H | Magneto P-lead sense | Magneto pulse input | Through 10kΩ to optocoupler input; 10kΩ isolating resistor preserved |

# 3. Bill of Materials

Approximate total cost for a single build: $55–95 USD depending on sourcing. Using JLCPCB for a custom PCB reduces assembly complexity significantly.

| Component / Description | Qty | Approx Cost | Source |
| --- | --- | --- | --- |
| MICROCONTROLLER |  |  |  |
| Arduino Nano (ATmega328P, 5V, 16MHz) — recommended for build accessibility | 1 | $4–8 | Amazon / AliExpress |
| Alt: STM32F103C8T6 'Blue Pill' — recommended for production, more timers, 3.3V |  |  |  |
| RPM INPUT / MAGNETO ISOLATION |  |  |  |
| 6N137 High-Speed Optocoupler (recommended over PC817 — faster rise time, cleaner pulse edge) | 1 | $0.50 | Mouser / DigiKey |
| 1/4W Resistor 10kΩ (input current limit for optocoupler LED — matches original isolating resistor value) | 1 | $0.05 | Any |
| 1/4W Resistor 4.7kΩ (output pull-up for optocoupler) | 1 | $0.05 | Any |
| 0.1µF ceramic capacitor (noise bypass across optocoupler input) | 1 | $0.10 | Any |
| RELAY DRIVE (INC / DEC RPM OUTPUTS) |  |  |  |
| ULN2003A Darlington driver array — drives existing airframe relays from logic-level MCU output | 1 | $0.40 | Any |
| Note: Uses existing airframe relays (INC/DEC RPM). No new relays needed unless box is self-contained. |  |  |  |
| 1N4001 flyback diode (relay coil protection) — if driving any internal relay coils | 2 | $0.10 | Any |
| POWER SUPPLY |  |  |  |
| LM2596S-5V DC-DC step-down regulator module (14V aircraft bus → 5V, 3A capable) | 1 | $1.50 | AliExpress / Amazon |
| Note: Do NOT use 7805 linear regulator — dissipates ~1.2W as heat in a sealed box. Use switching regulator. |  |  |  |
| 100µF/25V electrolytic capacitor (input filter) | 1 | $0.20 | Any |
| 47µF/16V electrolytic capacitor (output filter) | 1 | $0.15 | Any |
| 0.1µF ceramic (bypass, across power rails near MCU) | 3 | $0.15 | Any |
| PANEL DISPLAY |  |  |  |
| SSD1306 OLED 1.3" 128x64 I2C (more readable than 0.96" for instrument panel) | 1 | $6–10 | Amazon / AliExpress |
| Alt: MAX7219 4-digit 7-segment LED — more sunlight-readable, no graphics; use if OLED washes out |  |  |  |
| PANEL CONTROLS |  |  |  |
| Rotary encoder with push switch (KY-040 or equivalent) — fine RPM trim +/- 10 RPM per detent | 1 | $1–2 | Amazon |
| Tactile pushbutton (momentary, panel-mount rated) — one per preset | 5 | $0.50 | DigiKey / Mouser |
| 10kΩ linear taper panel potentiometer (replaces original rheostat — same 0–10kΩ, correct taper) | 1 | $3–8 | Bourns PDB182 or Alpha series |
| CRITICAL: Must be LINEAR taper, not audio/log taper. This is likely the fix for non-linear sensitivity. |  |  |  |
| CONNECTORS & INTERFACE |  |  |  |
| 7-pin Cannon/MS-series plug — mates with existing airframe governor connector | 1 | $15–40 | Aircraft Spruce / B&C |
| 22AWG shielded 2-conductor cable (panel to governor box run, for display + controls) | ~6ft | $3 | Any electronics supply |
| 2.54mm pin header / socket strips (for modular MCU mounting — allows swap without soldering) | 2x20 | $1 | Any |
| PCB & ENCLOSURE |  |  |  |
| Prototype PCB 80x60mm (Eurocard size fits in original die-cast box footprint) | 1 | $1–3 | Amazon / AliExpress |
| Alt: Custom PCB via JLCPCB — $5 for 5 boards at this size if design is finalized |  |  |  |
| Standoffs M3 x 10mm nylon (PCB mounting inside box) | 4 | $0.50 | Any |
| PASSIVES & MISCELLANEOUS |  |  |  |
| 1/4W resistors assorted 1kΩ, 4.7kΩ, 10kΩ, 47kΩ (pull-ups, biasing, current limits) | 10ea | $1 | Any |
| 0.1µF ceramic capacitors (bypass / noise filtering throughout board) | 10 | $0.50 | Any |
| 10µF/16V electrolytic (supply decoupling near MCU) | 2 | $0.20 | Any |
| LED indicator 3mm red + green (motor INC/DEC activity indicator — useful for ground test) | 2ea | $0.30 | Any |
| DeoxIT D5 contact cleaner (for servicing panel rheostat connections) | 1 can | $12 | Amazon / hardware |
| Heat shrink tubing assorted | 1 pack | $3 | Any |
| 22AWG solid core wire assorted colors | 1 roll | $8 | Any |

Total Estimated Cost: $55–95 single build / $35–55 per unit in small quantities with custom PCB

# 4. RPM Preset Configuration

The five presets below reflect the requested operating points. These are stored in EEPROM and survive power cycles. All values are editable in firmware constants before flashing.

| Preset Name | Target RPM | Selector | Description & Notes |
| --- | --- | --- | --- |
| MAX RPM | 2650 | Full CW / Button 1 | Full fine pitch. Takeoff, go-around. Do not cruise at this setting — monitor CHTs. |
| CRUISE CLIMB | 2450 | Button 2 | Best-power climb after departure. Good CHT management. |
| CRUISE | 2300 | Button 3 (default) | Standard cruise. Default at power-on AUTO engagement. |
| ECONOMY CRUISE | 2200 | Button 4 | Long-range cruise. Verify power settings for desired fuel flow. |
| LOW / EMERGENCY | Full Coarse Limit | Full CCW / Button 5 | Engine failure drag reduction. Motor commanded to full coarse until limit switch opens. Removes governor from loop. |

Fine trim: The rotary encoder adjusts the active setpoint in ±10 RPM increments. Trim value displayed in real time and held until preset button is pressed again. Trim does not modify EEPROM — it's session-only.

# 5. Software Architecture

## 5.1 RPM Measurement

The magneto fires once per cylinder per 2 crankshaft revolutions. For a 6-cylinder Continental (IO-470/IO-520 etc.), this produces 3 pulses per revolution. For a 4-cylinder, it produces 2. This must be set as a firmware constant (PULSES_PER_REV).

// Firmware constant — set before flash #define PULSES_PER_REV 3 // 6-cyl Continental // Use 2 for 4-cylinder engines // Hardware interrupt on falling edge of optocoupler output volatile uint32_t lastPulseTime = 0; volatile uint32_t pulseInterval = 0; ISR(INT0_vect) { // or attachInterrupt() equivalent uint32_t now = micros(); pulseInterval = now - lastPulseTime; lastPulseTime = now; } // Convert to RPM with rolling average (8-sample buffer) float calculateRPM() { static uint32_t buf[8]; static uint8_t idx = 0; buf[idx++ & 7] = pulseInterval; uint32_t avg = 0; for (int i = 0; i < 8; i++) avg += buf[i]; avg /= 8; if (avg == 0) return 0; return 60000000.0 / (avg * PULSES_PER_REV); }

## 5.2 Control Algorithm

The core control loop runs every 100ms. The proportional stage reduces motor run time as the RPM approaches the setpoint, replacing the original full-voltage blast with a modulated pulse.

#define DEADBAND 25 // RPM — no action within ±25 of setpoint #define PROP_ZONE 75 // RPM — proportional slowdown begins within ±75 #define MAX_DRIVE_MS 8000 // Safety: max continuous relay time in ms #define MAG_TIMEOUT 2000 // Safety: alarm if no pulse for 2 seconds void controlLoop() { float actualRPM = calculateRPM(); float setpoint = getActiveSetpoint(); // from preset + encoder trim float error = setpoint - actualRPM; // Safety: no magneto signal if (millis() - lastPulseTime > MAG_TIMEOUT) { relayOff(INC_RELAY); relayOff(DEC_RELAY); displayAlarm("NO MAG SIGNAL"); return; } // Deadband — do nothing if close enough if (abs(error) < DEADBAND) { relayOff(INC_RELAY); relayOff(DEC_RELAY); return; } // Proportional zone — scale motor on-time float duty = 1.0; // full power by default if (abs(error) < PROP_ZONE) { duty = abs(error) / PROP_ZONE; // 0.33 to 1.0 in proportional zone duty = max(duty, 0.25); // minimum 25% — enough to move prop } if (error > 0) { // RPM too low — increase RPM (decrease pitch / fine) relayOff(DEC_RELAY); pulsedRelay(INC_RELAY, duty, MAX_DRIVE_MS); } else { // RPM too high — decrease RPM (increase pitch / coarse) relayOff(INC_RELAY); pulsedRelay(DEC_RELAY, duty, MAX_DRIVE_MS); } }

## 5.3 Proportional Relay Pulsing

The existing relays are not rated for PWM at audio frequencies. The proportional function is implemented as time-proportioned bursts within each 100ms control cycle — relay on for (duty × 100ms), then off for the remainder. This is well within relay contact ratings.

void pulsedRelay(uint8_t relay, float duty, uint32_t maxMs) { static uint32_t relayOnSince[2] = {0, 0}; uint32_t now = millis(); // Safety: cut relay if held too long without RPM change if (relayOnSince[relay] > 0 && (now - relayOnSince[relay]) > maxMs) { relayOff(relay); displayAlarm("RELAY TIMEOUT"); return; } uint32_t onTime = (uint32_t)(duty * 100); // ms on within 100ms cycle uint32_t cyclePos = now % 100; if (cyclePos < onTime) { if (relayOnSince[relay] == 0) relayOnSince[relay] = now; digitalWrite(relay, HIGH); } else { relayOnSince[relay] = 0; digitalWrite(relay, LOW); } }

## 5.4 Display UI

The OLED displays four lines of information updated at 5Hz. The display is intentionally simple — readable at a glance in turbulence.

┌──────────────────────────┐ │ CRUISE AUTO │ Line 1: Preset name + mode (AUTO/MAN) │ SET: 2300 RPM │ Line 2: Active setpoint (numeric) │ ACT: 2287 RPM │ Line 3: Actual measured RPM │ HOLD ──────────── │ Line 4: Status (HOLD / INC▲ / DEC▼ / ALARM) └──────────────────────────┘ When encoder trim is active: ┌──────────────────────────┐ │ CRUISE +30 AUTO │ Trim offset shown next to preset name │ SET: 2330 RPM │ Setpoint reflects trim │ ACT: 2318 RPM │ │ INC▲ ████░░░░░░ │ Bar graph shows motor duty cycle └──────────────────────────┘ Alarm state (no mag signal, relay timeout): ┌──────────────────────────┐ │ !! ALARM !! MAN │ Flashing header, reverts to MANUAL │ NO MAG SIGNAL │ Alarm message │ ACT: ---- │ RPM unknown │ CHECK RIGHT MAGNETO │ Diagnostic hint └──────────────────────────┘

## 5.5 Mode State Machine

States: MANUAL — relays de-energized, prop controlled by panel toggle switch only (same as original OFF/MANUAL operation — this is the safe default) AUTO — control loop active, presets and encoder active ALARM — control loop halted, relays de-energized, display shows fault Cleared only by cycling AUTO switch (requires pilot action) Transitions: Power-on → MANUAL (always — never auto-engage at power-on) AUTO switch ON → AUTO AUTO switch OFF → MANUAL Mag timeout → ALARM Relay timeout → ALARM Alarm + switch cycle → MANUAL (not directly back to AUTO)

## 5.6 EEPROM / Non-Volatile Storage

Stored in ATmega328P internal EEPROM (1KB — more than sufficient): Address 0x00: Magic byte (0xAE) — detects blank/corrupt EEPROM Address 0x01: Preset 1 RPM (uint16, 2 bytes) — MAX Address 0x03: Preset 2 RPM — CRUISE CLIMB Address 0x05: Preset 3 RPM — CRUISE (default on AUTO engage) Address 0x07: Preset 4 RPM — ECONOMY CRUISE Address 0x09: Preset 5 mode flag (0xFF = full coarse, else RPM target) Address 0x0B: Deadband value (uint8, in RPM) Address 0x0C: Proportional zone (uint8, in RPM) Address 0x0D: PULSES_PER_REV (uint8) — 2 or 3 Address 0x0E: Last active preset index (restored on next power-on) EEPROM is written only when values change, never in the control loop. ATmega328P EEPROM rated for 100,000 write cycles — no wear concern.

## 5.7 Firmware Build Environment

- Arduino IDE 2.x or PlatformIO (recommended for professional builds)

- Target: Arduino Nano (ATmega328P) or STM32 Blue Pill (with appropriate board package)

- Libraries: Adafruit_SSD1306, Adafruit_GFX (OLED), EEPROM (built-in), TimerOne (PWM/timing)

- Watchdog: avr/wdt.h (ATmega328P) — wdt_enable(WDTO_250MS) in setup(), wdt_reset() in main loop

- All safety-critical code (watchdog reset, relay cutoff) must execute within the 250ms watchdog window regardless of display update state

# 6. Safety Architecture Summary

| Failure Mode | Detection Method | Response |
| --- | --- | --- |
| Firmware crash / infinite loop | Hardware watchdog timer (250ms) | MCU reset → power-on safe state (MANUAL) |
| Loss of magneto signal | Pulse timeout > 2 seconds | Relays de-energize, ALARM state, display message |
| Relay stuck on / limit switch fail | Motor run-time counter > 8 seconds | Relay force-off, ALARM state |
| Power failure | N/A — passive | Relays de-energize (spring return) — same as original |
| Noisy magneto signal | 8-sample rolling average on pulse interval | Spurious pulses averaged out; no false corrections |
| Dirty/noisy panel pot | 10ms ADC averaging (50+ samples) | Noise rejected; smooth setpoint |
| MCU ADC offset / inaccuracy | Calibration constants in EEPROM | Applied at boot; field-adjustable via serial port |
| EEPROM corruption | Magic byte check at boot | Defaults loaded; operator warned on display |

⚠ REGULATORY NOTE: This design uses the existing airframe relays and limit switches as the final actuator elements. Relay contact current flows through the existing wiring, not through the governor box. The governor box outputs are relay coil drive currents only (~100mA per coil). This means the governor cannot physically overvoltage or reverse the prop motor on its own — the airframe wiring and limit switches remain the last line of defense, unchanged from the original installation.

Next Steps: Share this document with an IA familiar with Bonanza avionics. The key regulatory question is whether this qualifies as a replacement under the existing STC SA2700WE (same external interface, same function) or requires a new 337. Either path is supportable given the documentation available.